	Tender text

	

	[image: image1.png]



	
	

	
	
	Salvis Deep Fat Fryer

	
	
	SFR-EVO-42

	
	
	

	
	
	Freestanding execution with integrated pump system and basket lifting device 

	
	
	

	
	
	· Dual-pan deep fat fryer 

· Electronic control system with digital display

· 4 buttons for programmable temperature and time

· Frying basket included


	
	
	


Specific characteristics

· Dual-pan deep fat fryer for frying and baking of meat, fish, potatoes, vegetables, and pastries.
· Swivel round-tube heating element in the basin with an enlarged heating surface for more efficient heat

transfer, including a working and safety thermostat.

· Integrated fat/oil drain with shut-off function for convenient emptying of the basin contents.
· Double filter system consisting with easy-to-clean metal and cloth filters for efficient collection of frying


residues.

· Removable collection container made of stainless steel on ball-bearing slide rails enables safe and


convenient oil and fat maintenance.

· Basin insulation (R-value 0.18 m2K/W) for minimised energy loss.
Execution

· Powerful and heat resistant pump automatically transfers the filtered oil from the collection tray back into


the frying basin at the push of a button.
· Practical feed tap with plastic handle, quick-release coupling and heat-resistant hand hose makes it easy

to fill, rinse or pump out the frying fat.

· Automated basket lifting device for lowering and lifting the frying basket at the start and end of the frying

process.
Control:

· Simple, user-friendly interface with a high-quality membrane keypad seamlessly integrated into the


control panel.

· Digital display for optimal readability of temperature and time.

· Four buttons for easy selection of variable temperature and time settings.

· Automatic fat melting stage during the heating phase.

· Programmable fat/oil usage duration for optimal monitoring of fat lifespan.

· AQA – Automatic Quantity Adjustment: Intelligent quantity adaptation for optimal cooking results.

· Energy-saving function - automatic temperature reduction after 30 minutes of inactivity.
· Electronic timer with with start/stop button and acoustic signal.
· Fat temperature adjustable from 50 - 180 °C for increased performance and capacity.

· Precise temperature detection of +/-1 °C thanks to electronic temperature sensor.

Housing:

· Seamless and deep drawn and joint less welded basin with large scaled radiuses, for easy cleaning. 

· Wide foaming zone and drain downwards throughout a ball-valve.

· Closed base unit with hinged door, left-side hinge – easily changeable to the right.

· Adjustable CNS feet, 150 mm high.

· Complete unit, basin, and base unit cladding made of CNS 1.4301 stainless steel.

Documentation

· Installation and operating manual

Options and accessories:

· Mobile kit, 2 adjustable legs in front and 2 fixed castors on the rear, 2 pan, with 2 cable, 


with 2 plug type CEE 16 A

· Frying basket portion, 105 x 320 x 155 mm (2 baskets can be placed in one tank)

· Connecting path for hygienic and safe installation of 2 free-standing fryers

· Mamito - Automatic dosing system to extend the oil service life

Technical specification

	Useful content



Total content


Well dimension


Basket content

Hourly output chilled / frozen
	2 x 4.5 – 6.0 Litre 
2 x 7.5 – 9.0 Litre 
2 x 160 x 428 x 338 mm

2 x 1.25 kg

50 kg / 38 kg

	External dimension:

                        width

                        depth

                        height
	400 mm

650 mm

750/900 mm

	Weight
 (net / gross)

Connected load
	75 / 86 kg

2 x 8.0 kW

	Tension

Fuses

	400 V 3 50/60 Hz

2 x (3 x 16 A )

	Approvals

Protection
	CE

IPX4


Technical subjects to modification!

[image: image2.jpg]smart cooking



Version  02/2025
- 1 -

